# BUILD AND SECURE A MINI ENTERPRISE NETWORK

Capstone Project Report

Student Name: Rishitha Bandaru

Learner ID: 17947

Batch / Team: Cyber Security July 2026

Assessment Target: Ubuntu web server (Nginx 1.24.0) at 192.168.206.5 in a two-VM VirtualBox lab network

Assessment Date: 19 October 2026

Tools: Kali Linux, Ubuntu, VirtualBox, Nmap 7.95, Wireshark, UFW, curl

## 1. Objective and scope

This project built a small virtual network in VirtualBox, scanned it with Nmap, captured its traffic with Wireshark, and hardened the exposed service with a host firewall.

The lab contains two virtual machines: a Kali Linux machine used as the assessment host, and an Ubuntu machine running an Nginx 1.24.0 web server. Both sit on the 192.168.206.0/24 subnet. All results below come from this two-machine setup.

Tools used: VirtualBox, Kali Linux, Ubuntu, Nmap 7.95, Wireshark, UFW, curl and ping.

## 2. Network architecture

lab topology · 2 VMs on 192.168.206.0/24

Kali (192.168.206.4) scans and browses to the Ubuntu server (192.168.206.5), which serves a web page through Nginx on TCP 80. The firewall shown on Ubuntu is the UFW configuration applied during hardening (section 8). Ubuntu's second adapter (10.0.2.15) is shown for completeness.

## 3. IP address table

Both machines received their addresses dynamically on 192.168.206.0/24. The capture also shows a DHCP server at 192.168.206.2.

| Host | Role | Interface | IP address | Subnet | Assignment | MAC address |
| --- | --- | --- | --- | --- | --- | --- |
| Kali Linux | Assessment host | eth0 | 192.168.206.4 | /24 (255.255.255.0) | Dynamic (DHCP) | 08:00:27:36:4b:58 |
| Ubuntu | Web server (Nginx) | enp0s3 | 192.168.206.5 | /24 (255.255.255.0) | Dynamic (DHCP) | 08:00:27:28:76:45 |
| Ubuntu | Second adapter | enp0s8 | 10.0.2.15 | /24 (255.255.255.0) | Dynamic (DHCP) | not captured |
| DHCP server | Address assignment | n/a | 192.168.206.2 | /24 | n/a | 08:00:27:9b:22:7a |

Kali also lists a docker0 interface (172.17.0.1/16) in state DOWN (NO-CARRIER); it carries no lab traffic.

## 4. Configuration documentation

The two machines were configured, then checked for addressing, connectivity, web service and name resolution.

IP configuration. ip -4 addr on each machine confirms the addresses in section 3.

Connectivity. Kali pinged the Ubuntu server with ping -c 4 192.168.206.5: 4 packets sent, 4 received, 0% packet loss, round-trip min/avg/max/mdev 1.620 / 3.014 / 4.985 / 1.227 ms, TTL 64.

Web server. Nginx was enabled and started on Ubuntu with sudo systemctl enable --now nginx, and systemctl is-active nginx returned active.

From Kali, curl -I http://192.168.206.5 returned HTTP/1.1 200 OK with Server: nginx/1.24.0 (Ubuntu), Content-Type: text/html and Content-Length: 615.

DNS. On Ubuntu, getent hosts archive.ubuntu.com resolved the name to IPv6 addresses (for example 2620:2d:4002:1::102 and 2a06:bc80:0:1000::17), showing name resolution works.

## 5. Nmap results

The scan found one open port on the Ubuntu server: 80/tcp, running nginx 1.24.0.

Command run from Kali on 19 Oct 2026 (Nmap 7.95): nmap -sV -oN ~/Mini_Enterprise_Network/03_Nmap_Results/ubuntu_scan.txt 192.168.206.5

| Item | Result |
| --- | --- |
| Target | 192.168.206.5 (host up, 0.0032 s latency) |
| Ports scanned | Nmap's default 1000 TCP ports: 999 closed (reset), 1 open |
| Open port | 80/tcp, http, nginx 1.24.0 (Ubuntu) |
| MAC address | 08:00:27:28:76:45 (Oracle VirtualBox virtual NIC) |
| OS info | Linux (CPE: cpe:/o:linux:linux_kernel) |
| Scan time | 20.72 seconds, 1 host up |

## 6. Wireshark evidence

The capture shows the full HTTP exchange between Kali and Ubuntu in clear text.

Traffic was captured on Kali's eth0 and saved as lab_traffic.pcapng (20 packets).

| Packets | Protocol | What it shows |
| --- | --- | --- |
| 1–3 | TCP | Three-way handshake from 192.168.206.4:59498 to 192.168.206.5:80 (SYN, SYN-ACK, ACK) |
| 4–7 | HTTP / TCP | HTTP GET request (packet 4, 147 bytes) and the server's response (packet 6, 928 bytes) |
| 8–10 | TCP | Connection closed with FIN/ACK in both directions |
| 11–12 | mDNS | Multicast queries for _ipp._tcp.local (IPv6 and 224.0.0.251) |
| 13–16 | ARP | Kali and Ubuntu resolve each other's MAC addresses |
| 17–18 | DHCP | Request and Ack from 192.168.206.2 |
| 19–20 | ARP | Kali resolves 192.168.206.2 |

With the http display filter, two packets remain: the GET request from 192.168.206.4 to 192.168.206.5 and the reply. The request headers, including Host: 192.168.206.5 and a curl User-Agent, are readable in the packet bytes.

## 7. Security findings

The Ubuntu web server had no active firewall, and its only service ran over unencrypted HTTP with a visible version.

| # | Finding | Evidence |
| --- | --- | --- |
| 1 | Host firewall inactive: UFW reported Status: inactive before hardening, so nothing filtered inbound traffic on the Ubuntu server. | sudo ufw status on Ubuntu |
| 2 | Web traffic is unencrypted: the HTTP GET request and response crossed the network in clear text and were readable in Wireshark. Only port 80 was open in the scan. | Wireshark capture, http filter |
| 3 | Server version is disclosed: both the HTTP Server header and Nmap service detection reveal nginx/1.24.0 (Ubuntu). | curl -I output, Nmap -sV scan |
| 4 | Small exposed surface (positive): of 1000 TCP ports scanned, only 80/tcp was open and 999 were closed. | Nmap scan |

## 8. Hardening and recommendations

UFW was enabled with a default-deny inbound policy and a single allow rule for port 80, and the web server still responded afterwards.

Applied on the Ubuntu server:

- sudo ufw default deny incoming

- sudo ufw default allow outgoing

- sudo ufw allow 80/tcp

- sudo ufw enable

sudo ufw status verbose then showed Status: active, logging on (low), default deny (incoming) / allow (outgoing) / disabled (routed), and one rule: 80/tcp ALLOW IN from Anywhere (IPv4 and IPv6).

Verification. From Kali, curl -I http://192.168.206.5 still returned HTTP/1.1 200 OK from nginx/1.24.0 (Ubuntu) after the firewall was enabled.

Recommendations (suggested next steps, not carried out in this project):

- Serve the site over HTTPS (port 443 with a TLS certificate) so traffic is encrypted, addressing finding 2.

- Hide the Nginx version (server_tokens off;), addressing finding 3.

- Limit the port 80 rule to the hosts that need it, instead of Anywhere.

- Re-run the Nmap scan after hardening to confirm the exposed ports.

## 9. Conclusion

The lab network was set up with two virtual machines on 192.168.206.0/24, verified with ping, curl and DNS tests, scanned with Nmap, and captured with Wireshark. The assessment found an inactive firewall, unencrypted HTTP and a disclosed server version on the Ubuntu web server. Enabling UFW with default-deny inbound and an allow rule for port 80 closed the firewall gap while keeping the web service reachable.